MOSFET Characteristics

Nanotechnology and Nanoelectronics Engineering

Zewail City of Science, Technology and Innovation

Analog IC Design NANENG 332– Fall 2026

Youssef Ahmed Mohamed                                                                                                          ID: 202401212

# 1. Introduction

In this lab we characterize an NMOS transistor using Cadence Virtuoso. The device is the nch transistor of the 65 nm design kit, with Ln = 60 nm, Wn = 200 nm, Mn = 1 and Fn = 1 as the default values. We run DC sweeps on VGS and VDS and study how the channel length, the channel width, the number of fingers and the multiplier change the characteristics. After that we check the device across process, voltage and temperature (PVT) corners, and finally we look at the transconductance gm.

# 2. Task 1: DC Sweep on VGS

Figure 1 shows the testbench schematic.

Figure 1: Testbench schematic used to characterize the NMOS device.

## ID versus VGS and the threshold voltage

VGS was swept from 0 to 1.2 V with a step of 10 mV while making Vdd =800mV Figure 2 shows the resulting ID versus VGS curve, and from the graph we can estimate that VTH is about 0.35 V.

Figure 2: ID versus VGS at VDS = 800 mV.

We can estimate Vth = 0.4V from the graph

## Effect of the channel length

Here we define the parametric sweep of Ln, from 60n to 240n with 4 points (Figure 3). Figure 4 shows ID versus VGS for the different lengths. Clearly, increasing L reduces ID.

Figure 3: Parametric sweep setup for Ln.

Figure 4: ID versus VGS for different channel lengths.

## Effect of the channel width

After returning Ln back to 60n, we sweep Wn from 150n to 5u with a step count of 4 (Figure 5). Figure 6 shows ID versus VGS for the different widths. Increasing Wn increases ID.

Figure 5: Parametric sweep setup for Wn (150n to 5u, 4 points).

Figure 6: ID versus VGS for different channel widths.

## Effect of the number of fingers

Next we sweep Fn from 1 to 5 with a step count of 5 (Figure 7). Figure 8 shows VGS on the x-axis and ID on the y-axis for the different values of Fn. Increasing Fn increases ID.

Figure 7: Parametric sweep setup for Fn (1 to 5, 5 points).

Figure 8: ID versus VGS for different numbers of fingers.

## Effect of the multiplier

Figure 9 shows ID versus VGS for different values of Mn, from 1 to 5 with a step count of 5. Increasing Mn also increases ID.

Figure 9: ID versus VGS for different multipliers (Mn = 1 to 5).

## Question

Is it possible to sweep the gate thickness?

No. In physical manufacturing, the gate oxide thickness (Tox) is set during wafer fabrication. The analog designer does not design vertical parameters like Tox. The designer only controls the lateral dimensions (W and L), the number of fingers and the multiplier.

# 3. Task 2: DC Sweep on VDS

Here we sweep VDS instead of VGS. VDS is swept from 0 to 1.2 V with a 10 mV step (Figure 10), and VGS is set to VTH + 300 mV = 650 mV (Figure 11).

Figure 10: DC analysis setup: VDS swept from 0 to 1.2 V with a 10 mV step.

Figure 11: Design variables with VGS set to 0.65 V.

## ID versus VDS and the operating regions

Figure 12 shows ID versus VDS. We can clearly see that the triode (linear) region goes from VDS = 0 V to about 0.15 V, and from 0.15 V up to 1.2 V (the end of the sweep) the device is in the saturation region. This is lower than the long-channel estimate VGS − VTH = 0.3 V, because velocity saturation makes a short-channel device saturate earlier. At VGS = 650 mV the transistor is above threshold along the whole sweep, so there is no cut-off region on this curve. The cut-off region appears when VGS is below VTH, as in Figure 14.

We can also see from the graph that the transistor is not fully saturated. This is due to channel length modulation: ID depends on Leff, and Leff depends on VDS, so there is still some dependence between ID and VDS in the saturation region. In a 60 nm device, DIBL also adds to this effect.

Figure 12: ID versus VDS at VGS = 650 mV.

## Effect of VGS

Now we sweep VGS as well, from 0 to 1.2 V (Figure 13). Figure 14 shows ID versus VDS for the different values of VGS. When VGS is smaller than VTH, like the red (0 V) and yellow (0.3 V) curves, there is no ID regardless of VDS, because there are not enough charge carriers to make the channel conduct. Increasing VGS increases the number of charge carriers in the channel. Once VGS becomes bigger than VTH, as in the other curves, the current starts to flow, and a higher VGS means more charge carriers and hence a higher ID.

Figure 13: Parametric sweep setup for VGS.

Figure 14: ID versus VDS for different values of VGS.

## Effect of the channel length

We sweep Ln again, now with the VDS sweep (Figure 15). Figure 16 shows ID versus VDS for the different values of Ln, and we clearly see the inverse proportionality between ID and L.

Figure 15: Parametric sweep setup for Ln (VDS sweep).

Figure 16: ID versus VDS for different channel lengths.

## Effect of the width, fingers and multiplier

Figure 17, Figure 18 and Figure 19 show the results of sweeping Wn, Fn and Mn respectively. In the three cases ID increases as the parameter increases.

Figure 17: ID versus VDS for different channel widths.

Figure 18: ID versus VDS for different numbers of fingers.

Figure 19: ID versus VDS for different multipliers.

## Question

What is the difference between the results when sweeping on Fn, Mn and Wn?

In standard DC simulations, sweeping the width (Wn), the number of fingers (Fn) or the multiplier (Mn) will all increase the drain current ID. Mn places identical copies of the transistor in parallel, and Fn splits the transistor into fingers inside one device, so in our results Fn and Mn give almost the same curves (ID is multiplied by the sweep factor). Wn makes the transistor itself wider, and here it covers a much bigger range (150 nm to 5 µm), so the change in ID is much large.

# 4. Task 3: PVT Corner Analysis

In this task we check the device across process, voltage and temperature corners. The process corners are fast, typical and slow NMOS (FF, TT, SS), the voltage corners are VDD + 10%, VDD and VDD − 10% (1.32 V, 1.2 V and 1.08 V), and the temperature corners are hot, room and cold (125 °C, 27 °C and −40 °C). The corners were set up as shown in Figure 20.

We made the fast corner happen at the high temperature and the high supply, the typical corner at room temperature and the slow corner at the low temperature. A higher temperature increases the intrinsic carrier concentration, which lowers the Fermi potential and therefore lowers VTH (typically by around 1 to 2 mV per °C), so the fast corner ends up with the lowest threshold voltage.

Figure 20: Corners setup: FF (125 °C, 1.32 V), TT (27 °C, 1.2 V) and SS (−40 °C, 1.08 V).

## ID versus VGS and the threshold voltage

Figure 21 shows ID versus VGS for the different cases: FF at high temperature, TT at room temperature and SS at low temperature, each with a different VDD. This lets us check the limits of our device before sending it for manufacturing. The FF corner gives the highest current and the SS corner gives the lowest.

Figure 21: ID versus VGS across the PVT corners (FF, TT, SS).

Different temperature, VDD and process combinations correspond to different values of VTH. We calculated VTH using a function in the Cadence calculator, and Figure 22 shows the results: 156.1 mV for FF, 347.4 mV for TT and 498.8 mV for SS (the nominal value is 373.8 mV).

Figure 22: VTH of each corner calculated with the Cadence calculator.

## ID versus VDS

Figure 23 shows ID versus VDS at a fixed VGS for the three corners. It shows how the device behaves at the extremes of the PVT range, with FF again giving the highest current and SS the lowest.

Figure 23: ID versus VDS at fixed VGS across the PVT corners.

## Questions

What is the difference between the results when sweeping on Fn, Mn and Wn?

Sweeping Mn or Fn: the current ID is multiplied by the sweep factor, and the threshold voltage VTH stays practically the same across all PVT corners.

Sweeping Wn: the current also increases, but this changes the geometry of the device itself, This causes VTH to shift slightly across the corners.

Search for short channel effects and give a brief about what you understood.

Short channel effects appear when the channel length becomes comparable to the depletion widths of the source and drain junctions. The main ones are:

- Velocity saturation and mobility degradation. In our derivation of ID we assumed that the drift velocity is v = µE. At high electric field the carrier velocity saturates and is no longer linear with E. Because of this, ID stops following the square law and the device saturates earlier.

- DIBL (drain induced barrier lowering). A high drain voltage pulls down the energy barrier at the source side of the channel. This lowers the threshold voltage and makes ID depend more on VDS.

- Loss of gate control. Because the gate’s grip is weakened by the drain’s influence, the transistor never fully turns off and the subthreshold leakage current increases. In addition, the very thin gate oxide of these devices allows quantum mechanical tunneling from the gate to the channel, which is a separate source of gate leakage.

In our simulations these effects show up as the incomplete flattening of ID in the saturation region (Figure 12) and the early saturation of the curves.

# 5. Task 4: gm Characteristics

In this task we study the transconductance gm. First we plot VTH against VGS (Figure 24). It is just a graph between VGS and VTH, and we clearly see that VTH changes very slightly with VGS (from about 365 mV to 377 mV), so we can treat it as a constant.

Figure 24: VTH versus VGS.

## gm versus (VGS − VTH) at constant W/L

Figure 25 is the graph between VGS and gm, but we want VGS − VTH. For that we made a function in the calculator that subtracts VTH from VGS (Figure 26), and the result is shown in Figure 27.

Figure 25: gm versus VGS.

Figure 26: Calculator expression used to plot gm against (VGS − VTH).

Figure 27: gm versus (VGS − VTH) at constant W/L.

For VGS − VTH below zero the transistor is in weak inversion and gm is very small. Above VTH, gm rises quickly as the transistor goes into strong inversion. In the ideal square-law model, gm = µCox(W/L)(VGS − VTH) would keep growing linearly, but here the curve bends and flattens at about 300 µS, and it even drops slightly at the end. This is the effect of velocity saturation and mobility degradation in the short channel.

## gm versus ID at constant W/L

We use the calculator again to plot gm against ID (Figure 28). The result is shown in Figure 29.

Figure 28: Calculator setup used to plot gm against ID.

Figure 29: gm versus ID at constant W/L.

gm grows quickly with ID at low current, where the device is in weak inversion and gm ∝ ID. As ID increases the slope drops (the square law predicts gm ∝ √ID), and gm finally levels off at about 300 µS, where velocity saturation takes over and gm no longer depends on ID.

## gm versus (VGS − VTH) at constant ID

At constant ID, the only way to change VGS is to change W/L, so every point needs its own width. We tried two approaches.

### Method 1: square-law width scaling

First, we swept Wn to find the width that gives ID = 1 mA at a few values of VGS (Table 1). The smaller the overdrive, the wider the device must be.

Table 1: Width required for ID = 1 mA at different values of VGS.

| VGS (V) | 1.200 | 1.017 | 0.833 | 0.650 |
| --- | --- | --- | --- | --- |
| Wn (µm) | 1.00 | 1.32 | 1.95 | 3.56 |

To automate this over a full sweep, we kept Ln fixed and scaled only Wn, since width is the easiest parameter to adjust in practice. From the square law,

ID = ½ µnCox (Wn/Ln) (VGS − VTH)2

and holding ID equal to its value at a reference point (VGS = 0.8 V, width Wref) gives

Wn = Wref · [ (0.8 − VTH) / (VGS − VTH) ]2

with VTH ≈ 0.4 V. We entered this expression as the design variable of Wn (rounded to a 5 nm grid) and swept VGS from 0.8 V to 1.2 V in 10 mV steps (Figure 30). Starting from Wref = 200 nm would give only 50 nm at 1.2 V, which is outside the model's range, and Spectre stopped with error CMI-2440. We raised Wref to 600 nm, so Wn runs from 600 nm down to 150 nm.

Figure 30: Width expression defined as a design variable of Wn.

Figure 31 shows that ID is not constant: it falls from about 268 µA at 0.8 V to about 158 µA at 1.2 V, a drop of 41%. In this short-channel device, velocity saturation makes ID grow closer to linearly with VGS − VTH (an effective exponent of about 1.2–1.3 instead of 2), so the formula shrinks the width too much. VTH also drifts slightly with VGS (Figure 24). With ID swinging between roughly 160 and 270 µA, treating it as constant would be a poor approximation, so square-law scaling is not reliable here. At the widths we did simulate, gm falls from about 870 µS at VGS − VTH = 0.4 V to about 230 µS at 0.8 V. This follows the expected 2ID/(VGS − VTH) trend but stays below it.

Figure 31: ID versus VGS with Wn scaled by the square-law expression.

Figure 32: gm versus VGS -Vth with Wn scaled by the square-law expression.

### Method 2: nested sweep and gm/ID

The ratio gm/ID does not depend on width, so we can simulate once and rescale to any target current instead of resizing the device each time. We ran a nested sweep with VGS from 0 to 1.2 V (20 steps, plus 0.8 V) and VDS from 0.6 to 1.2 V (20 steps), using Wn = 200 nm, Ln = 60 nm, Fn = Mn = 1 and VTH = 0.35 V (Figure 32). The ID–VDS data was exported as idvsvds.csv and processed in MATLAB (full script in Appendix A).

Figure 33: ID versus VDS for the nested VGS sweep.

The script picks a saturation point (VDS = 0.9 V), computes gm = ∂ID/∂VGS across the traces, and rescales it to a target current of Itarget = 100 µA:

gm(ID = Itarget) = Itarget · (gm / ID)

Figure 33 plots the result against VGS − VTH together with the square-law curve 2ID/(VGS − VTH).

Figure 34: gm versus (VGS − VTH) at constant ID = 100 µA.

gm decreases steadily as the overdrive grows: about 2.7 mS in weak inversion, 1.0 mS at VGS − VTH ≈ 0.1 V, 0.3 mS at 0.4 V and 0.13 mS at 0.85 V. This matches Method 1: at the same current, a small overdrive gives more gm at the cost of a wider transistor. The simulated curve sits below 2ID/(VGS − VTH) because of velocity saturation and mobility degradation. The two diverge near threshold, where the square-law expression blows up while the real gm/ID levels off at about 27 V−1. Because Method 2 uses simulated data and assumes no square law, we consider it the more reliable of the two.

# 6. Conclusion

In this lab we characterized a 65 nm NMOS transistor with DC sweeps in Cadence. VTH came out at about 0.35–0.4 V from the ID–VGS curve (373.8 mV from the simulator at the nominal corner). ID decreases with L and increases with W, Fn and Mn. The ID–VDS curves showed cut-off, triode and saturation, and ID was not perfectly flat in saturation because of channel-length modulation and DIBL.

Across PVT corners, FF gave the highest current and the lowest VTH (156.1 mV), while SS gave the lowest current and the highest VTH (498.8 mV). gm levels off near 300 µS because of velocity saturation.

At constant ID, gm falls as the overdrive rises. Square-law width scaling failed to hold ID constant (it dropped by 41%), whereas the nested-sweep gm/ID method gave a reliable result that lies below the square-law prediction, again because of short-channel effects.

# Appendix A: MATLAB Script

% MATLAB code: gm vs (VGS - VTH) at constant ID

% Input file: idvsvds.csv (Id vs Vds, 21 Vgs traces). Change I_target and Vth at the top.

clc; clear; close all;

%% Settings

I_target = 100e-6;      % constant Id (A)

Vth      = 0.35;        % threshold voltage (V)

Vds_fix  = 0.9;         % keep it in saturation

%% Load (col 1 = Vds, even columns = Id for each Vgs trace)

raw = readmatrix('D:\Cadence\idvsvds.csv', 'NumHeaderLines', 3);

Vds = raw(:,1);

Id  = raw(:,2:2:end);                    % rows = Vds, cols = Vgs

Vgs = sort([linspace(0,1.2,20) 0.8]);    % 21 traces, legend order

assert(size(Id,2) == numel(Vgs), 'Vgs list does not match number of traces');

%% gm = dId/dVgs across the traces, at the chosen Vds

[~, r] = min(abs(Vds - Vds_fix));

id = Id(r,:);

gm = gradient(id, Vgs);

%% gm at constant Id (W scaled so every point carries I_target)

gm_const = I_target * gm ./ id;

Vov      = Vgs - Vth;

keep = Vov > -0.1;                       % drop deep subthreshold

%% Plot (clean light style, dark readable text)

fig = figure('Color','w','Position',[100 100 820 580]);

theme(fig,'light');                      % R2025a+ : forces light theme

ax = axes(fig); hold(ax,'on'); box(ax,'on');

plot(ax, Vov(keep), gm_const(keep)*1e3, '-o', 'Color',[0 0.25 0.75], ...

'LineWidth',2, 'MarkerSize',6, 'MarkerFaceColor',[0 0.25 0.75]);

Vp = linspace(0.05, max(Vov), 100);

plot(ax, Vp, 2*I_target./Vp*1e3, '--', 'Color',[0.85 0.1 0.1], 'LineWidth',1.8);

ax.Color      = 'w';

ax.XColor     = 'k';

ax.YColor     = 'k';

ax.GridColor  = [0.3 0.3 0.3];

ax.GridAlpha  = 0.25;

ax.FontName   = 'Arial';

ax.FontSize   = 12;

ax.LineWidth  = 1.2;

grid(ax,'on');

xlabel(ax, 'V_{GS} - V_{TH} (V)', 'FontSize',14, 'FontWeight','bold', 'Color','k');

ylabel(ax, 'g_m (mS)',            'FontSize',14, 'FontWeight','bold', 'Color','k');

title(ax, sprintf('g_m vs (V_{GS} - V_{TH}) at constant I_D = %.0f \\muA', I_target*1e6), ...

'FontSize',15, 'FontWeight','bold', 'Color','k');

lgd = legend(ax, 'Simulated', '2I_D / V_{ov} (square law)', 'Location','northeast');

lgd.TextColor = 'k';  lgd.Color = 'w';  lgd.EdgeColor = [0.5 0.5 0.5];  lgd.FontSize = 12;

% Optional: save a high-resolution image for the report

% exportgraphics(fig, 'gm_vs_vov.png', 'Resolution', 300);